Four-Fold Cross-Validation: PINN-LSTM vs. LSTM Baseline
IGBT RUL Reproduction of Lu et al. (2023), NASA PCoE Dataset #8

Leave-one-device-out  |  MSE ×10⁻³  |  lower MSE / higher R² = better  |  ΔMSE>0 and ΔMSE%>0 mean PINN beat baseline

| Test device | LSTM MSE | LSTM R² | PINN-LSTM MSE | PINN-LSTM R² | ΔMSE (abs) | ΔMSE (%) | Lu PINN MSE | Lu PINN R² | ¦dev¦ MSE | ¦dev¦ R² |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 9.48 | 0.877 | 9.28 | 0.880 | 0.20 | +2.1% | 7.29 | 0.881 | 2.00 | 0.001 |
| 3 | 5.95 | 0.922 | 8.33 | 0.891 | -2.38 | -40.0% | 1.65 | 0.978 | 6.68 | 0.087 |
| 4 | 17.06 | 0.778 | 19.99 | 0.740 | -2.93 | -17.2% | 5.96 | 0.898 | 14.03 | 0.158 |
| 5 | 3.71 | 0.952 | 5.35 | 0.930 | -1.64 | -44.2% | 4.09 | 0.954 | 1.27 | 0.024 |
| AVG | 9.05 | 0.882 | 10.74 | 0.860 | -1.69 | -18.6% | 4.75 | 0.930 | 5.99 | 0.070 |

Caption. On this run the physics-informed LSTM did not improve on the vanilla LSTM baseline: averaged across the four folds its test MSE was 18.6% higher (worse) and mean R² fell from 0.882 to 0.860 — the opposite of the +13.9% MSE improvement Lu et al. (2023) report for PI-LSTM. The vanilla-LSTM baseline R² values are nonetheless reasonable (mean 0.882), confirming the recurrent model fits the degradation signal once the paper architecture (80-cell single-layer LSTM, seq=10) is used. The largest deviation from Lu et al. is on the device-4 fold (|ΔMSE| = 14.0×10⁻³, |ΔR²| = 0.158), where both models fit worst. [FILL IN after inspecting device 4: the most likely driver is the automatic end-of-life detection (FAILURE_DROP_THRESHOLD) and resulting RUL labels on this device's noisy late-life V_ce; confirm whether the detected N_f matches the paper's hand-identified failure point, and note the average-downsampling window vs the paper's per-square-wave-cycle averaging.] All runs use the Lu-matching configuration (80-cell single-layer LSTM, sequence length 10, α=0.1, β=100, γ=0.1, Adam, 2000 epochs) under leave-one-device-out cross-validation on NASA PCoE Dataset #8. The physics terms add little here because the monotonic-decreasing penalty sees only small violations under heavy window overlap, while the β=100 boundary penalty can distort training when predictions stray slightly outside [0,1]. Lu et al. PI-LSTM reference values are read from their Figure 10 (×10⁻³).

Generated from outputs/four_fold_comparison.csv. Lu et al. (2023), Sci. Rep. 13:10167 (CC BY 4.0); reference values from Figure 10.